NOTE TO READERS: The following is a supplementary document, detailing calculations and offering additional context and information for the main paper (above). The two documents are merged for reviewers but the supplementary document is separate after publication.

This paper is designed mostly to detail and argue for the methods employed in a larger project. Consequently, most of the data, and specifics, are contained within that project. This supplementary document therefore does not cover, for example, our estimates of on-field agricultural energy use, nor the employment of fertilizers over time—and certainly not our estimates of the broader energy system. However, we did offer estimates of the food system more specifically in this methods paper than we have before; these estimates are detailed below.

Combining these estimates produces the following chart of agricultural use with embodied energy in pesticides, fertilizer, human labor, and animal feed; the overall conclusions of our paper remain surprisingly robust:

Figure S.1: Combined estimates of embodied energy in U.S. agriculture, 1840-2008: This chart combines the additional estimates detailed throughout the rest of this supplemental document. Note we class horses, mules, and asses as “working” here even though they are generally hobby animals in the late twentieth century.

Animal Feed

Using a similar method as we did for our estimates of horse feed, it is possible to estimate inputs of livestock feed across agriculture. We do not count these in our main estimates, as detailed above, in part to facilitate comparability with other sectors; it is also in part because the feed itself is a product of agriculture, which renders this still less comparable to other sectors. Nevertheless, this might be informative. Census and USDA reports make it possible to estimate the evolution of the live weight of stock over the course of American history, as we chart here:

| year | calves | yearlings | mature steers |  | mature cows |  | hogs |  | chickens |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | average | average | low | high | low | high | low | high | average | source |
| 1850 |  |  | 216 |  |  |  |  |  |  | 1880 census |
| 1880 |  |  | 352 | 476 |  |  | 78 | 205 |  | 1880 census |
| 1923 |  |  | 500 | 517 |  |  |  |  |  | Wilcox et al. |
| 1925 |  |  |  |  |  |  | 41 | 106 |  | Robbins |
| 1928 |  |  |  |  | 386 | 417 | 114 | 136 |  | Wilson et al |
| 1930 |  |  | 505 | 534 | 338 | 465 |  |  |  | Semple and Dvorachek |
| 1931 |  |  | 414 | 639 |  |  |  |  |  | Black et al (1931) |
| 1931 |  |  | 510 | 655 |  |  |  |  |  | Burmeister et al |
| 1933 |  |  |  |  |  |  | 69 | 157 |  | Ellis |
| 1933 |  |  |  |  |  |  | 60 | 73 |  | Nighburt |
| 1933 |  |  | 435 | 514 |  |  |  |  |  | Woodward and Graves |
| 1933 |  |  |  |  | 558 | 573 |  |  |  | Dawson et al. |
| 1934 |  |  |  |  |  |  | 32 | 94 |  | Ellis and Zeller |
| 1934 |  | 407 |  |  |  |  |  |  |  | Black et al. (1934) |
| 1938 |  |  |  |  |  |  | 100 |  |  | Bjorka |
| 1938 |  |  |  |  | 458 | 505 |  |  |  | Black et al (1938) |
| 1939 | 294 |  |  |  |  |  |  |  |  | Wahlenberg |
| 1943 |  |  | 364 | 428 |  |  |  |  |  | Black et al (1943) |
| 1943 |  |  | 360 | 448 |  |  |  |  |  | McComas et al |
| 1945 |  |  |  |  |  |  | 95 |  |  | Atkinson and Klein |
| 1945 | 500 | 518 |  | 561 |  |  |  |  |  | Nelson |
| 1946 | 263 |  |  | 644 |  |  |  |  |  | Hankins and Howe |
| 1949 | 240 |  |  |  | 365 | 468 |  |  |  | Black |
| 1952 | 231 |  |  |  |  | 486 |  |  |  | Converse and Wiseman |
| 1954 | 253 | 323 |  |  | 466 | 580 |  |  |  | Matthews and Fohrman (1954a) |
| 1954 |  | 398 |  |  |  | 455 |  |  |  | Matthews and Fohrman (1954b) |
| 1956 |  |  |  |  |  | 550 |  |  |  | Shepherd |
| 1957 |  |  |  |  |  |  |  |  | 0.905 | Zuidhof et al. |
| 1957 |  |  | 445 | 596 |  |  |  |  |  | Winchester and Ellis |
| 1958 |  |  |  |  |  | 523 |  |  |  | Clark |
| 1961 |  |  |  |  | 473 | 523 |  |  |  | Reed and Peterson |
| 1966 |  |  |  |  | 420 | 489 |  |  |  | Houston and Woodward |
| 1967 |  |  | 455 |  |  |  |  |  |  | Winchester |
| 1976 |  |  |  |  |  |  | 82 | 136 |  | Sullivan et al |
| 1978 |  |  |  |  |  |  |  |  | 1.808 | Zuidhof et al. |
| 1985 |  |  |  |  |  |  | 107 |  |  | Arsdall and Nelson |
| 2008 |  |  |  |  |  |  |  |  | 4.2 | Zuidhof et al. |

Table 1: Livestock Mass estimates, 1850-2020. These summarize the estimates of various American government officials, farmers, and scientists as to the average weight of various non-working stock. There is an overall moderate positive trendline between all these estimates, though goats, hogs, and sheep appear to have only very modestly grown over the time of the study.

We use this to estimate the total weight of all livestock over time:

Figure S.2: Estimated mass of livestock (kg), 1840-2015: This chart uses the estimated weights of livestock from Table S.1, multiplied by the number of livestock from NASS/Agricultural Census, to arrive at total estimated mass of livestock.

It is particularly noteworthy that, on a per capita basis, the energy of livestock feed has seen extremely limited change; that is, roughly as much energy is used on a per person basis to feed livestock to supply the meat of Americans in 1890 as in 2020. If anything, it has somewhat declined in line with wider agricultural energy use:

Figure S.3: Estimated Livestock Feed: This chart estimates the energy embodied in livestock feed in Watts/capita. Working stock feed are taken from our prior estimates. For non-working stock, we multiply contemporary studies of livestock feeding requirements by historical masses of livestock.

Of course, on an absolute scale, the number has increased in line with the American population. But it is remarkable that the amount of energy to supply the average American with meat is so stable. The quality and species of meat consumed by the average American has dramatically shifted, indicating some plasticity. The relative decline of pork is obvious, as is the recency of the widespread availability of chicken. But large-scale changes to the American diet seem fairly limited over time, with deep cultural roots that might present serious barriers to reducing the energy requirements for agriculture. It is also notable from a historical perspective that the American agricultural system was so meat-heavy even in the period of 1840, with its fairly limited levels of industrialization, interconnectivity in transport, or fossil fuel use.

Fertilizers and Pesticides

It is also possible to include, to a limited extent, the embodied energy in fertilizers and pesticides applied on field. In our larger estimates, the energy required in their manufacture are included under the category of “industry” rather than agriculture. By far, the largest quantity of fertilizer used is artificial nitrogen, with artificial phosphorous and other fertilizers lagging far behind; pesticides are likewise a surprisingly minimal flow, though they appear to be increasing over time for a variety of reasons.

Figure S.4: On field agricultural energy with embodied energy from fertilizer and pesticides, 1800-2009. This chart includes the embodied energy from the active ingredients in pesticides as well as from nitrogen fertilizer. (Artificial phosphate fertilizers have been omitted as their embodied energy contents are negligible; see above.)

Human Labor

We include here an estimate of the human labor in agriculture, as this has traditionally been a sector employing significant amounts of manual labor. Despite this, as we noted in the main paper, the relative quantity of energy employed in human labor is a fairly tiny quantity. Even if 100% of the population were employed in the agricultural system, they would not have comprised more than a quarter of the total in the early nineteenth century. For the most part, in the nineteenth century, horses were employed for the heavy labor of farm work (as well as many industrial processes like hauling timber, ore, or construction materials), while humans were employed primarily for tasks requiring dexterity—e.g., the raking and cleaning of reapers, on-field repairs, binding wheat, or feeding threshers. They are still employed for tasks which have no easy mechanized means, as in the delicate process of fruit or vegetable harvest. The below estimate includes human labor, with total number of laborers taken from the Census to 1930 and the NBER thereafter:

Figure S.5: Energy in Agricultural Human Labor, 1840-2020. This chart displays the Watts/capita expended by human laborers in agriculture, assuming approximately 150 Watts/capita per laborer (roughly equivalent to a 3,000 kcal/day diet of a hard laborer). Note that these flows are quite low compared to virtually any other agricultural input. Note that the absolute number of agricultural workers has declined since roughly 1900 with the advent of mechanization.

Industrial Food Systems

As explained in the main paper, detailed industrial fuel use was surveyed by the 1910 Census and thereafter. The overall use of energy in food systems is therefore fairly easy to trace back to 1910 (and exhibits hardly any change on a per capita basis over the ensuing century). Extrapolating it back to earlier periods is significantly more challenging. We are able to break the food system’s industrial energy use down into specific sub-industries, and then track the development of those back in time to the beginning of our data set—1800.

The component industries of the food system in 1910 were liquor distillation and breweries, ice manufacturing, slaughterhouses and meat packing, flour and gristmills, bakeries, sugar refining, cottonseed oil refining, salt manufacture, dairies, and canneries (sorted largest to smallest); we first calculate the quantity of energy used by each of these according to the 1910 Census (Table S.1). The proxies used for tracking these are displayed below (Tables S.2-S.7).

|  | Anthracite (long tons) | Bituminous (short tons) | Coke (Short Tons) | Wood (cords) | Oil (barrels) | Gas (1000 ft3) | Water (hp) | Total (Watts) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Liquors | 554029 | 2424798 | 11530 | 69637 | 527089 | 3649380 | 340 | 2442670510 |
| Manufactured Ice | 244396 | 2430454 | 6872 | 183984 | 905952 | 4348929 | 2262 | 2368128098 |
| Meat-Packing | 193935 | 2512864 | 976 | 37603 | 1102550 | 1091164 | 46 | 2197434195 |
| Flour/Gristmills | 67459 | 1880702 | 3347 | 202748 | 424386 | 3037008 | 264131 | 1788614271 |
| Bakery Products | 383390 | 250165 | 195971 | 310933 | 151371 | 2732911 | 334 | 1116726656 |
| Sugar Refining | 511640 | 542148 | 362 | 85 | 578633 | 453 | 240 | 902721706 |
| Cottonseed Oil/Cake | 8583 | 828357 | 800 | 156447 | 376789 | 372880 | 175 | 784923942 |
| Salt | 108816 | 642373 | 15 | 9441 | 85074 | 1688731 | 108 | 620485284 |
| Ag. Implements | 15114 | 550085 | 98819 | 14528 | 244759 | 313412 | 8390 | 615414860 |
| Dairy | 47344 | 588169 | 1069 | 182587 | 94325 | 215318 | 1465 | 607286996 |
| Beet Sugar | 8981 | 565672 | 28300 | 3602 | 554174 | 383 | 200 | 554638168 |

Table S.1: Fuels used by the food industry in 1910. Each are converted into Watts using physical assumptions for fuels. These values are taken from the 1910 Census Report on Manufactures.

Tracking liquor production in the United States is actually a fairly easy process, as this is one of the few items that was taxed early on by domestic production as opposed to via an import or export tariff. We make no assumptions about the changing production efficiency of liquors (as we are interested in use of energy over time this would be assuming the answer to the question in the initial premises). What is notable is that liquor is quite energy intensive per unit produced, and that the amount of liquor produced early in the United States is extremely high on a per capita basis. Whiskey (also whisky, and often though not always bourbon) in particular was a highly durable, easily transportable, valuable good, preserving grain calories in a form that would last effectively indefinitely—and, of course, delicious and intoxicating. Other distilled spirits listed in these reports were produced in minimal quantities, and are therefore ignored here.

|  | distilleries | gallons distilled | watts employed | W/c | sources |
| --- | --- | --- | --- | --- | --- |
| 1810 | 14191 | 25804792 | 4.15E+08 | 57.32724 | A Statement of the Arts and Manufactures of the United States of America, for the year 1810, 22 |
| 1850 | 1217 | 47864724 | 7.7E+08 | 33.19493 | 1850 Census: Compendium of the Seventh Census, 182 |
| 1860 | 2741 |  |  |  |  |
| 1870 | 3089 | 76339807 | 1.23E+09 | 31.84377 | Annual Report of the Commissioner of Internal Revenue for the Fiscal Year Ended 1900, 129 |
| 1880 | 3152 | 90484356 | 1.46E+09 | 28.99714 | Ibid. |
| 1890 | 1688 | 1.1E+08 | 1.78E+09 | 28.19897 | Ibid. |
| 1900 | 2476 | 1.07E+08 | 1.72E+09 | 22.61408 | Ibid. |
| 1910 | 2027 | 1.52E+08 | 2.44E+09 | 26.43297 | Annual Report of the Commissioner of Internal Revenue for the Fiscal Year Ended 1910, 36 |

Table S.2: Liquor distillation, 1800-1910. We do not use the number of distilleries here, but they are included for general interest, as, like most industrial products, the quantity produced increases massively while the number of facilities declines precipitously, indicating a centralization of the production process. Watts employed is taken in 1910 from table S.2, and extrapolated backwards using the quantity of whisky distilled (column 2). These are then divided by the population of the United States to provide watts/capita (W/c).

Ice was one of the more difficult parts of the food industry to measure. This is both because ice was barely tracked as a commodity, and because before the manufacture of ice, natural ice was cut from lakes and rivers during winter for use in storage, transport, and the home. Ice cut from rivers in New York, Massachusetts, and Maine were sold in large cities, to the south, and even internationally, while it is safe to assume locals used it extensively across the rural north (where the bulk of the population lived). A contemporary estimated that the quantity of ice consumed in 1886 was roughly 25 million tons of natural cut ice—multiplying by the latent heat of fusion (the amount of energy needed to melt ice), this works out to roughly 9 watts/capita. But the ratio of natural ice to artificial ice is hard to determine, especially over time. Given the paucity of evidence, these are some of the least certain numbers possible—it would not be at all surprising if this number were closer to constant over the course of the nineteenth century.

Meatpacking production was among the most famous—and brutal—industrial food processes. It was also one of the few with essentially no analogue before the mid-nineteenth century, as commercial beef production exploded after the colonization of the Great Plains and the creation of a beef cattle industry out West. We scale the 1910 quantity of energy used according to the horsepower installed in facilities (available back to 1890), and between 1880 and 1890 by the quantity of fresh beef produced. At only 5 W/c in 1880, it is sufficiently low to be ignored in years prior.

|  | Meatpacking Plants | hands employed | pounds beef sold, fresh | horsepower employed | watts employed | W/c | sources |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1880 | 872 | 27207 | 7.59E+08 |  | 2.41E+08 | 4.7924 | Remarks on the Statistics of Manufactures, xvii; Statistics of Manufactures for Principal Industries, 474 |
| 1890 | 611 | 40409 | 1.63E+09 | 46738 | 4.92E+08 | 7.8135 | Motive Power Used in Manufactures, 766; Manufacturing Industries, 730-735 |
| 1900 | 497 | 76637 |  | 87060 | 9.17E+08 | 12.047 | 1910 Census, Vol 8, Manufactures, 337 |
| 1910 |  |  | 4.21E+09 | 208707 | 2.2E+09 | 23.779 | 1910 Census, Vol 8, Manufactures, 337, 379 |

Table S.3: Energy used in meatpacking, 1880-1910. Note again the number of plants and hands employed—indicating the centralization of industry.

Flour and gristmills are here scaled by the power employed in them between 1870 and 1910, and then by the amount of wheat produced before this. This is actually quite likely an overestimate of earlier power employed—much of the horsepower installed in mills in 1910 was steam-powered, which was far less efficient (likely 5-10%) than the waterwheels earlier mills would have used (which was bookkeep as 100% efficient).

|  | Wheat produced | power employed in milling | watts employed | W/c | sources |
| --- | --- | --- | --- | --- | --- |
| 1840 | 85000000 |  | 2.72E+08 | 15.94482 | Historical Statistics of the United States |
| 1850 | 1E+08 |  | 3.2E+08 | 13.80652 | Historical Statistics of the United States |
| 1860 | 1.73E+08 |  | 5.54E+08 | 17.62793 | Manufactures, Totals of, 1860, 736; Report on Agriculture, xxix, xlii, xlvi |
| 1870 | 2.88E+08 | 576686 | 9.21E+08 | 23.89518 | 1880 Census, Vol 2, Report on Power Used in Manufactures, 4; Special Statistics of Manufacturing, 599 |
| 1880 |  | 771201 | 1.23E+09 | 24.54973 | Ibid. |
| 1890 | 4.68E+08 | 752365 | 1.2E+09 | 19.0861 | 1890 Census, Vol 6, Motive Power Used in Manufactures, 757; Vol 6, Manufacturing Industries, 702; Agriculture, 11, 14 |
| 1900 |  | 1016859 | 1.62E+09 | 21.35122 | 1900 Census, Vol 7, Power Employed in Manufactures, cccxxvii |
| 1910 |  | 1119508 | 1.79E+09 | 19.3552 | 1910 Census, Vol 8, Manufactures, 335; this lists 1899 as 670719 and 1909 as 853584 |

Table S.4: Energy used in flour and gristmills. 1840-1910.

Sugar refining here is scaled by pounds of sugar produced—this generally from cane sugar, as beet sugar was virtually nonexistent before 1910, and cane sugar significantly outweighed maple sugar for all years covered. Here again we scale by production statistics. The result is a fairly even production except for the middle of the century, when production in the region of southern Louisiana suffered significantly during the end of the Civil War. Commonwealths are not included under these statistics, omitting the archipelago of Hawai’i.

|  | pounds of sugar produced | watts employed | W/c | sources |
| --- | --- | --- | --- | --- |
| 1810 | 7867211 | 10291767 | 1.421538 | A Statement of the Arts and Manufactures of the United States of America, for the year 1810, 26 |
| 1840 | 1.55E+08 | 2.03E+08 | 11.88676 | 1850 Census: Compendium of the Seventh Census, 174 |
| 1850 | 2.37E+08 | 3.1E+08 | 13.35798 | ibid |
| 1860 | 2.31E+08 | 3.02E+08 | 9.6099 | Agriculture of the United States in 1860; Compiled from the Original Returns of the Eighth Census, xcix |
| 1870 | 87043000 | 1.14E+08 | 2.953142 | The Statistics of the Wealth and Industry of the United States, 85 |
| 1880 | 1.79E+08 | 2.34E+08 | 4.66231 | Report on the Productions of Agriculture, 212 |
| 1890 | 3.01E+08 | 3.94E+08 | 6.258132 | Report on the Productions of Agriculture, 403 |
| 1900 | 4.66E+08 | 6.1E+08 | 8.016127 | Report on the Productions of Agriculture, 463 |
| 1910 | 6.9E+08 | 1.46E+09 | 15.77059 | Report on the Productions of Agriculture, 685 |

Table S.5: Energy used in sugar refining, 1810-1910.

Cottonseed oil began to be extracted from cotton in the late 1850s as a multipurpose oil. Its refining was tracked only beginning in the 1870s, and we use the total quantity produced to scale the energy used.

|  | Cottonseed production (thousand tons) | watts employed | W/c |
| --- | --- | --- | --- |
| 1870 | 1786 | 3.16E+08 | 8.18487 |
| 1880 | 2822 | 4.99E+08 | 9.935636 |
| 1890 | 3802 | 6.72E+08 | 10.66744 |
| 1900 | 4500 | 7.95E+08 | 10.45042 |
| 1910 | 4442 | 7.85E+08 | 8.493929 |

Table S.6: Energy used in cottonseed oil refining, 1870-1910. The non-energy data is sourced entirely from Carter, et al., Historical Statistics of the United States.

Finally, salt production is used to scale the use of energy in salt-making, primarily heat to boil water, although much of the process of salt production often involved the concentration of natural seawater in solar-evaporated beds that, for obvious reasons, are virtually impossible to quantify.

|  | salt production (ktons) | watts employed | W/C |
| --- | --- | --- | --- |
| 1880 | 757 | 1.23E+08 | 2.447367 |
| 1890 | 1017 | 1.65E+08 | 2.620194 |
| 1900 | 2503 | 4.06E+08 | 5.33761 |
| 1910 | 3824 | 6.2E+08 | 6.714482 |

Table S.7: Energy used in salt production, 1880-1910. Once again, all non-energy data is sourced from Carter, et al., Historical Statistics of the United States.

Between them, these industries account for roughly 85% of the total industrial energy used in the food system in 1910. The main exception here is bakeries and baked goods, which are omitted for a reason detailed in the main paper: the further back in time, the more baking was a domestic task, leading into the world of the ambiguous boundary between a stove being used for food preparation versus for warmth when both activities are happening simultaneously.

[STUDY details anonymized for review]

Modern weights taken from National Agricultural Statistics Service, “USDA National Statistics Services,” U.S. Department of Agriculture, Accessed March 1, 2024, Copyright 2024, https://www.nass.usda.gov/index.php, and all mentions of cattle, sheep, and hog weight from the United States Census Bureau, Tenth Census of the United States, Vol. 3: Report on the Productions of Agriculture (Washington, D.C.: Government Printing Office (hereafter GPO), 1884), as well as those listed in: R. H. Wilcox, R. D. Jennings, G. W. Collier, W. H. Black, and E. W. McComas, Costs and methods of fattening beef cattle in the corn belt, 1919-1923, USDA TB 23 (GPO: 1927), E. T. Robbins, “Cheaper and more profitable pork thru swine sanitation,” University of Illinois Central College and Experiment Station (Urbana, Ill., 1925), M. L. Wilson, R. H. Wilcox, G. S. Klemmedson, and V. V. Park, A study of ranch organization and methods of range-cattle production in the northern Great Plains region, USDA TB 45 (GPO: 1928), A. T. Semple and H. E. Dvorachek, Beef production from purebred, grade, and native calves, USDA TB 203 (GPO: 1930), W. H. Black, K. F. Warner, and C. V. Wilson, Beef production and quality as affected by grade of steer and feeding grain supplement on grass, USDA TB 217 (GPO: 1931), C. A. Burmeister, H. M. Conway, and A. P. Brodell, Economic factors affecting the beef-cattle industry of Virginia, USDA TB 237 (GPA: 1931), N. R. Ellis, Changes in Quantity and Composition of Fat in Hogs Fed a Peanut Ration Followed by a Corn Ration, USDA TB 368, (GPO: 1933), E. M. Nighbert, Effectiveness of the Swine Sanitation System in the South, USDA TB 374 (GPO: 1933), T. E. Woodward and R. R. Graves, Some results of inbreeding grade Guernsey and grade Holstein - Friesian cattle, USDA TB 339 (GPO: 1933), J. R. Dawson, A. G. Van Horn, and R. R. Graves, Sudan grass as hay, silage, and pasture for dairy cattle, USDA TB 352 (GPO: 1933), N. R. Ellis and J. H. Zeller, Effect of Quantity and Kinds of Feed on Economy of Gains and Body Composition of Hogs, USDA TB 413, (GPO: 1934), W. H. Black, A. T. Semple, and J. L. Lush, Beef production and quality as influenced by crossing Brahman with Hereford and Shorthorn cattle USDA TB 417 (GPO: 1934), Knute Bjorka, Shrinkage and Dressing Yields of Hogs, USDA TB 621, (GPO: 1938), W. H. Black, J. R. Quesenberry, A. L. Baker, Wintering beef cows on the range with and without a supplement of cottonseed cake, USDA TB 603 (GPO: 1938), W. G. Wahlenberg, H. R. Reed, and S. W. Greene, Effects of fire and cattle grazing on longleaf pine lands, as studied at McNeill, Miss., USDA TB 683 (GPO: 1939), W. H. Black, P. E. Howe, J. M. Jones, and F. E. Keating, Fattening steers on milo grain in the southern Great Plains USDA TB 847 (GPO: 1943), E. W. McComas, J. R. Douglas, B. L. Southwell, Corn-molasses mixtures compared with corn for fattening beef cattle in the Coastal Plain area, USDA TB 864 (GPO: 1943), L. Jay Atkinson and John W. Klein, Feed Consumption and Marketing Weight of Hogs, USDA TB 894 (GPO: July 1945), A. G. Nelson, Relation of feed consumed to food products produced by fattening cattle, USDA TB 900 (GPO: 1945), O. G. Hankins, P. E. Howe, Estimation of the composition of beef carcasses and cuts, USDA TB 926, (GPO: 1946), W. H. Black, Comparison of methods of supplying phosphorus to range cattle, USDA TB 981 (GPO: 1949), H. T. Converse and H. G. Wiseman, Corn silage as the sole roughage for dairy cattle, USDA TB 1057 (GPO: 1952), C. A. Matthews and M. H. Forhman, Beltsville growth standards for Holstein cattle, USDA TB 1099 (GPO: 1954), J. B. Shepherd, Permanent pasture compared with a 5-year crop-and-pasture rotation for dairy cattle feed, USDA TB 1144 (GPO: 1956), M.J. Zuidhof, B.L. Schneider, V.L. Carney, D.R. Korver, F.E. Robinson, “Growth, efficiency, and yield of commercial broilers from 1957, 1978, and 2005,” Poultry Science Volume 93, Issue 12, 2014, Pages 2970-2982, ISSN 0032-5791, https://doi.org/10.3382/ps.2014-04291, C. F. Winchester and N. R. Ellis, Delayed growth of beef cattle, USDA TB 1159 (GPO: 1957), R. T. Clark, Production factors in range cattle under northern Great Plains conditions, USDA TB 1181 (GPO: 1958), M. J. Reed and R. A. Peterson, Vegetation, soil, and cattle responses to grazing on northern Great Plains Range USDA TB 1252 (GPO: 1961), W. R. Houston and R. R. Woodward, Effects of stocking rates on range vegetation and beef cattle production in the northern Great Plains, USDA TB 1357 (GPO: 1966), C. F. Winchester, Malnutrition of young cattle; effect on feed utilization, eventual body size, and meat quality, USDA TB 1374 (GPO: 1967), James Dwight Sullivan, Charles Y. Liu, and Warren Vincent, A Systems Analysis of the Hog-Pork Subsector, USDA TB 1535, (GPO: 1976), Roy N. Van Arsdall and Kenneth E. Nelson, Economies of Size in Hog Production, USDA TB 1712, (GPO: 1985).

For the feeding requirements of various animals, see: P. J. Guiroy, D. G. Fox, L. O. Tedeschi, M. J. Baker, M. D. Cravey, “Predicting individual feed requirements of cattle fed in groups,” Journal of Animal Science, Volume 79, Issue 8, August 2001, Pages 1983–1995, https://doi.org/10.2527/2001.7981983x (cattle and other red meat); M. H. M. R. Fernandes, K. T. Resende, L. O. Tedeschi, J. S. Fernandes, H. M. Silva, G. E. Carstens, T. T. Berchielli, I. A. M. A. Teixeira, L. Akinaga, Energy and protein requirements for maintenance and growth of Boer crossbred kids, Journal of Animal Science, Volume 85, Issue 4, April 2007, Pages 1014–1023, https://doi.org/10.2527/jas.2006-110 (goats); Henry L. Classen, “Diet energy and feed intake in chickens,” Animal Feed Science and Technology, Volume 233, 2017, Pages 13-21, ISSN 0377-8401, https://doi.org/10.1016/j.anifeedsci.2016.03.004 (chickens); BA Shalev, H Pasternak, “The relative energy requirement of male vs female broilers and turkeys,” Poultry Science, Volume 77, Issue 6, 1998, Pages 859-863, ISSN 0032-5791, https://doi.org/10.1093/ps/77.6.859 (chickens and turkeys); Velayudhan DE, Kim IH, Nyachoti CM. Characterization of dietary energy in Swine feed and feed ingredients: a review of recent research results. Asian-Australas J Anim Sci. 2015 Jan;28(1):1-13. doi: 10.5713/ajas.14.0001R. PMID: 25557670; PMCID: PMC4283177 (hogs), suggesting diets of roughly 20Mcal/day or 20,000 kcal/day for cattle, 1,000 kcal/day for goats, 2,667 kcal/day for sheep, 5,500 kcal/day for swine, 350 kcal/day for chickens, and 1150 kcal/day for turkeys. Our prior estimates suggest horses burn roughly 36Mcal/day or 36,000 kcal/day for horses, and similar for mules and asses. Note that these are averages; most livestock raised for meat production are constantly changing size and diet over the course of their lives.

Jorge Fernandez-Cornejo, Richard Nehring, Craig Osteen, Seth Wechsler, Andrew Martin, and Alex Vialou, “Pesticide Use in U.S. Agriculture: 21 Selected Crops, 1960-2008,” Economic Research Service, Economic Information Bulletin Number 124, May 2014, M. B. Green, “Energy in pesticide manufacture, distribution and use,” in: Z.R. Helsel (editor), Energy in Plant Nutrition and Pest Control (Amsterdam: Elsevier, 1987), p.165-177; E. Audsley, K. Stacey, D.J. Parsons, and A.G. Williams, “Estimation of the greenhouse gas emissions from agricultural pesticide manufacture and use,” Cranfield University, Published 2009, Accessed March 1 2024, https://dspace.lib.cranfield.ac.uk/bitstream/handle/1826/3913/Estimation_of_the_greenhouse_gas_emissions_from_agricultural_pesticide_manufacture_and_use-2009.pdf.

Deborah Fitzgerald, Every Farm a Factory: The Industrial Ideal in American Agriculture (Yale University Press, 2010).

J. C. Jones, America's icemen: An illustrative history of the United States natural ice industry, 1665-1925, Jobeco Books, 1984.