S1 Appendix: Formal Description of Partition Constraints

Suppose a total of N measurements are taken over the course of the scan: denote these as Y=y1,…,yN. Choose a reconstruction temporal resolution such that we have T time points in the reconstructed image. We may identify each measurement with the time tn∈{1,…,T} at which it was taken. For example, if we partition the time duration of the scan into T intervals, then tn=1 indicates the nth measurement was acquired during the first time interval. We will also write vn∈{1,…,V} to denote thek-space voxel (i.e., spatial frequency) at which measurement n was acquired. Our method does not require that we observe an equal number of measurements during each time interval t=1,…,T, but we typically expect to have NT measurements for each t.

Let X=X1,…,XT be a dynamic timeseries image with T time-points, where each static image Xt has V voxels; this is the unknown sequence of true images (up to spatiotemporal discretization) that we aim to reconstruct. In a fully sampled regime (N=VT), we would observe the complete k-space data Xt=FXt at each time t=1,…,T, where F is the V×V discrete Fourier transform matrix. In other words, we would measure (a noisy version of) the sequence

X=X1,…,XT=FX1,…,FXT=IT⊗FX

where ⊗ denotes the Kronecker product. (Abusing notation, we will interpret X and X as either V×T matrices or vectors of length VT, depending on the context.) When reconstructing at accelerated time resolution (N<VT), the measured data is a proper subset of the full timeseries. Define

Ω=t1,v1,…,tN,vN

as the sequence of (time-index, spatial frequency-index) pairs at which the k-space measurements y1,…,yN were taken (i.e., each entry in Ω lies in the set 1,…,T×1,…,V). Then XΩ is the observed part of the Fourier transform of the dynamic image sequence. For each n, our observation yn is the voxel vn from the image Xtn, giving the relation yn=Xtnvn+noise. The remaining entries in X (i.e., those in XΩc) are left unobserved and must be reconstructed by our algorithm. Our data fidelity constraint stipulates that the observed k-space data Y must remain unaltered by the reconstruction. Therefore, any reconstruction  X of X must satisfy

IT⊗FXΩ=YΩ.1