S2 Appendix: The Penalty Function

After requiring that each reconstructed image match the k-space points in the partition corresponding to its interval, we minimize over a weighted and regularized smoothness penalty to determine a unique reconstruction. The mathematical details of this process are laid out in this section.

First, we will define the loss function minimized by our reconstruction optimization: the discretized curvature. For a given signal x=x1,…,xT composed of T time points, define the smoothness penalty function S as the l2-norm of its discrete second derivative:

Sx=t=2T-1xt+1-xt-xt-xt-12.

Since S is a quadratic function of x, we may write it in terms of a linear operator D satisfying

Sx=x*Dx.

As a matrix, D takes the form

The operator D is poorly conditioned. In fact, it can be shown that the respective supremum and infimum of eigenvalues of D (over all values of T) are σmaxD=16 and σminD=0.

Since D is (nearly) degenerate and we will later need to invert it, we add a small parameter λ (we chose λ=10-5) to regularize the operator D:

Dλ=D+λIT,

where IT is the T-dimensional identity. Thus, in the reconstruction process, we use the regularized smoothness penalty

Sλx=x*Dλx.

If X=X1,…,XT is a dynamic image composed of T static images, each with V voxels, then we can extend Sλ to act on X by

SλX=v=1Vxv*Dλxv,

where xv is the timeseries signal at the vth voxel.

It may also be desirable to penalize a lack of smoothness in some voxels more than in others. To enforce such a prioritization of “interesting” voxels, we may also include voxel-wise weighting terms wv, generating our full loss function L:

LX=v=1Vwvxv*Dλxv.

Setting W as the V×V diagonal matrix with entries wv, we can write the reconstruction optimization problem as

X=argminX W,XDλX* IT⊗FXΩ=YΩ.#2