| Table S-1: Radiomic feature names and descriptions. |  |  |
| --- | --- | --- |
| Image Feature | Feature Description | Reference |
| Volume (mm3) | Volume of lesion | Gilhuijs, K. G., Giger, M. L. and Bick, U., “Computerized analysis of breast lesions in three dimensions using dynamic magnetic-resonance imaging,” Med. Phys. 25(9), 1647–1654 (1998). |
| Effective diameter (mm) | Greatest dimension of a sphere with the same volume as the lesion |  |
| Surface area (mm2) | Lesion surface area |  |
| Maximum linear size (mm) | Maximum distance between any 2 voxels in the lesion |  |
| Sphericity | Similarity of the lesion shape to a sphere |  |
| Irregularity | Deviation of the lesion surface from the surface of a sphere |  |
| Surface area/volume (1/mm) | Ratio of surface area to volume |  |
| Margin sharpness | Mean of the image gradient at the lesion margin |  |
| Variance of margin sharpness | Variance of the image gradient at the lesion margin |  |
| Variance of radial gradient histogram | Degree to which the enhancement structure extends in a radial pattern originating from the center of the lesion |  |
| Contrast | Location image variations | Chen, W., Giger, M. L., Li, H., Bick, U. and Newstead, G. M., “Volumetric texture analysis of breast lesions on contrast-enhanced magnetic resonance images,” Magn. Reson. Med. 58(3), 562–571 (2007). |
| Correlation | Image linearity |  |
| Difference entropy | Randomness of the difference of neighboring voxels’ gray-levels |  |
| Difference variance | Variations of difference of gray-levels between voxel-pairs |  |
| Energy | Image homogeneity |  |
| Entropy | Randomness of the gray-levels |  |
| Inverse difference moment (homogeneity) | Image homogeneity |  |
| Information measure of correlation 1 | Nonlinear gray-level dependence |  |
| Information measure of correlation 2 | Nonlinear gray-level dependence |  |
| Maximum correlation coefficient | Nonlinear gray-level dependence |  |
| Sum average | Overall brightness |  |
| Sum entropy | Randomness of the sum of gray-levels of neighboring voxels |  |
| Sum variance | Spread in the sum of the gray-levels of voxel-pairs distribution |  |
| Sum of squares (variance) | Spread in the gray-level distribution |  |
| Maximum enhancement | Maximum contrast enhancement | Chen, W., Giger, M. L., Lan, L. and Bick, U., “Computerized interpretation of breast MRI: investigation of enhancement-variance dynamics.,” Med. Phys. 31(5), 1076–1082 (2004). |
| Time to peak (s) | Time at which the maximum enhancement occurs |  |
| Uptake rate (1/s) | Uptake speed of the contrast enhancement |  |
| Washout rate (1/s) | Washout speed of the contrast enhancement |  |
| Curve shape index | Difference between late and early enhancement |  |
| Enhancement at first postcontrast time point | Enhancement at first post-contrast time point |  |
| Signal enhancement ratio | Ratio of initial enhancement to overall enhancement |  |
| Volume of most enhancing voxels (mm3) | Volume of the most enhancing voxels |  |