# Supplementary materials:

# Detailed assumptions and model input data

The baseline assumptions and model input values listed in the following pages reflect the farm system design work of the ARPA-E MARINER teams, which include the authors of this paper, supported by Advanced Research Projects Agency-Energy (ARPA-E) Macroalgae Research Inspiring Novel Energy Resources (MARINER) award # DE-AR0000912 at the Marine Biological Laboratory, award # DE-AR0000915 at the Woods Hole Oceanographic Institution, and award # DE-AR0000911 at the University of Alaska.

## Supplementary table S1. Baseline assumptions for cool temperate farm

| Subsystem | Component |  | Parameter |  | Units |  | Baseline value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Farm module |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Grow rope | Length | m | 183 |  |
|  |  | Spacing | m | 0.75 |  |
|  |  | Cost | $ per m | 0.20 |  |
|  |  | Life | # harvests | 10 |  |
|  | Catenary | Span | m | 91.5 |  |
|  |  | Cost | $ per m | 0.16 |  |
|  |  | Life | years | 8 |  |
|  | Anchor | # per module | # | 8 |  |
|  |  | Life | years | 20 |  |
|  |  | Cost | $ per anchor | 300 |  |
|  | Mooring chain | Length | m per anchor | 15 |  |
|  |  | Cost | $ per m | 0.92 |  |
|  |  | Life | years | 20 |  |
|  | Mooring line | Scope | X:1 | 3 |  |
|  |  | Cost | $ per m | 0.11 |  |
|  |  | Life | years | 10 |  |
|  | Framing lines | Length | m | 550 |  |
|  |  | Cost | $ per m | 0.08 |  |
|  |  | Life | years | 8 |  |
|  | Corner hardware | # per module | # | 4 |  |
|  | (spar buoy) | Cost | $ | 3,000 |  |
|  |  | Life | years | 20 |  |
|  | Buoyancy | Cost | $ | 800 |  |
|  |  | Life | years | 10 |  |
|  | Installation (contract) | Per module | $ | 2,500 |  |
|  | Maintenance | Boat time/harvest | hours | 12 |  |
|  |  | Materials | $ per year | 2,000 |  |
|  | Removal (contract) | Per module | $ | 1,000 |  |
|  |  | Scrap value | $ | 500 |  |
| Farm operations |  |  |  |  |
|  | Vessel | Capital cost | $ | 5,000,000 |  |
|  |  | Life | years | 25 |  |
|  |  | Power plant | kW installed | 500 |  |
|  |  |  | kW used on farm | 200 |  |
|  |  | Fuel cons. | L h-1 per 100 kW | 20 |  |
|  |  | Diesel price | $ per L | 0.60 |  |
|  |  | OPEX | $ per hour | 5.00 |  |
|  |  | Crew | # | 4 + 4 |  |
|  |  | Operating speed | Knots | 7 |  |
|  |  | Biomass capacity | wet tonnes | 200 |  |
|  |  | Biomass discharge | tonnes per min | 5 |  |
|  | Crew | Wages | $/hour | 17.00 |  |
|  |  |  | Night work premium | 25% |  |
|  |  | P&I insurance | $/month/person | 500 |  |
|  | Harvesting gear | Capital cost | $ (installed) | 50,000 |  |
|  |  | Capacity | # of grow lines | 5 |  |
|  |  | OPEX | $ per hour | 1.00 |  |
|  |  | Life | years | 15 |  |
|  |  | Harvest speed | m s-1 | 0.5 |  |
|  | Seeding gear | Capital cost | $ (installed) | 25,000 |  |
|  |  | Capacity | # of grow lines | 5 |  |
|  |  | OPEX | $ per hour | 0.50 |  |
|  |  | Life | years | 15 |  |
|  |  | speed | m s-1 | 0.5 |  |
|  | Working shifts | Time between port calls | days | 7 |  |
|  |  | Port call duration | hours | 5 |  |
|  |  | Operating time | hours per day | 22 |  |
|  | Transitions | Between modules | minutes | 10 |  |
| Biology |  |  |  |  |  |
|  | Nursery | Seed production | $ per m total cost | 0.05 |  |
|  | Growout | Time to harvest | days | 180 |  |
|  |  | Planting window | days | 45 |  |
|  |  | Harvest window | days | 45 |  |
|  | Yield | Wet biomass | $ per m | 15 |  |
|  |  | Water content | % wet weight | 80% |  |
| Farm parameters |  |  |  |  |  |
|  | Location | Distance from shore base | km | 75 |  |
|  |  | Water depth | m | 50 |  |
|  |  | Current | m s-1 | 0.5 |  |
|  |  | Weather days | days per week | 1 |  |
|  | Scale | Farmed footprint | ha | 1,000 |  |
| Overhead |  |  |  |  |  |
|  | Staff | Wages | $ per hour | 25.00 |  |
|  |  | FTEs onshore | # | 4 |  |
|  | Office | Rent | $ per year | 50,000 |  |
|  | Docks | Rent | $ per year | 10,000 |  |
|  | Utilities | Annual cost | $ per year | 20,000 |  |
|  | Insurance | Annual cost | $ per year | 100,000 |  |
|  | Miscellaneous | Annual cost | $ per year | 10,000 |  |

## Supplementary table S2. Baseline assumptions for tropical farm

| Subsystem | Component | Parameter | Units | Baseline value |  |
| --- | --- | --- | --- | --- | --- |
| Farm module |  |  |  |  |
|  | Grow rope/net | Length | m | 300 |  |
|  |  | Spacing | m | 1.00 |  |
|  |  | Cost | $ per m | 0.30 |  |
|  |  | Life | # harvests | 16 |  |
|  | Catenary | Span | m | 50 |  |
|  |  | Cost | $ per m | 0.16 |  |
|  |  | Life | years | 8 |  |
|  | Buoyancy | Cost | $ per module | 500 |  |
|  |  | Life | years | 10 |  |
|  | Installation (contract) | Per module | $ | 2,500 |  |
|  | Maintenance | Boat time | hours per year | 8 |  |
|  |  | Materials | $ per year | 3,000 |  |
|  | Removal (contract) | Per module | $ | 1,000 |  |
|  |  | Scrap value | $ | 500 |  |
| Farm array (5x4 modules) |  |  |  |  |  |
|  | Dimensions | Growline axis | # of modules | 5 |  |
|  |  | Catenary axis | # of modules | 4 |  |
|  | Spacing | Growline axis | m between arrays | 100 |  |
|  |  | Catenary axis | m between arrays | 200 |  |
|  | Anchors | # per 5x4 array | # | 14 |  |
|  |  | Life | years | 20 |  |
|  |  | Cost | $ per anchor | 1,500 |  |
|  | Mooring chain | Length | m per anchor | 15 |  |
|  |  | Cost | $ per m | 10.00 |  |
|  |  | Life | years | 20 |  |
|  | Mooring line | Scope | x:1 | 3 |  |
|  |  | Cost | $ per m | 2.00 |  |
|  |  | Life | years | 10 |  |
|  | Framing lines | Length | m | 8,700 |  |
|  |  | Cost | $ per m | 0.30 |  |
|  |  | Life | years | 8 |  |
|  | Corner hardware | # per 5x4 array | # | 30 |  |
|  | (spar buoys) | Cost | $ | 3,000 |  |
|  |  | Life | years | 20 |  |
| Farm operations |  |  |  |  |
|  | Vessel | Capital cost | $ | 1,000,000 |  |
|  |  | Life | years | 25 |  |
|  |  | Power plant | kW installed | 300 |  |
|  |  |  | kW used on farm | 150 |  |
|  |  | Fuel cons. | L h-1 per 100 kW | 20 |  |
|  |  | Diesel price | $ per liter | 0.60 |  |
|  |  | OPEX | $ per hour | 5.00 |  |
|  |  | Crew | # | 4 |  |
|  |  | Operating speed | knots | 15 |  |
|  | Crew | Wages | $ per hour | 17.00 |  |
|  |  |  | Night work premium | 25% |  |
|  |  | P&I insurance | $/month/person | 500 |  |
|  | Harvesting gear | Capital cost | $ (installed) | 7,000 |  |
|  |  | Capacity | # of grow lines | 10 |  |
|  |  | OPEX | $ per hour | 0.50 |  |
|  |  | Life | years | 15 |  |
|  |  | Harvest speed | m s-1 | 0.5 |  |
|  | Planting gear | Capital cost | $ (installed) | 5,000 |  |
|  |  | Capacity | # of grow lines | 5 |  |
|  |  | OPEX | $ per hour | 0.20 |  |
|  |  | Life | years | 15 |  |
|  |  | Planting speed | m s-1 | 0.5 |  |
|  | Working shifts | Operating time | hours/day | 16 |  |
|  | Transitions | Between lines/ modules | minutes | 10 |  |
|  | Logistics | Crew shuttle | $/person per km | 0.10 |  |
|  |  | Supply shuttle | $/tonne to farm | 10 |  |
|  | Harvest aggregation | Drone tug (DT) | $ capital cost | 100,000 |  |
|  |  | DT fuel cons. | L h-1 | 5 |  |
|  |  | DT OPEX | $ per hour | 1 |  |
|  |  | DT life | years | 10 |  |
|  |  | DT towing speed | m s-1 full bag | 1.0 |  |
|  |  |  | m s-1 empty bag | 2.0 |  |
|  |  | Harvest bag | Ccpacity m3 | 100 |  |
|  |  |  | $ cost | 5,000 |  |
|  |  |  | life years | 1 |  |
|  |  | Harvest slurry | % water | 50% |  |
| Biology |  |  |  |  |  |
|  | Nursery | Seed production | $ per m total cost | 0.01 |  |
|  | Planting | Biomass | wet kg m-1 | 0.5 |  |
|  | Growout | Time to harvest | weeks | 9 |  |
|  | Yield | Biomass | wet kg m-1 | 5.5 |  |
|  |  | Water content | % wet weight | 86% |  |
| Farm parameters |  |  |  |  |  |
|  | Location | Distance from shore base | km | 75 |  |
|  |  | Water depth | m | 50 |  |
|  |  | Current | m s-1 | 0.5 |  |
|  |  | Weather days | days per year | 30 |  |
|  | Scale | Farmed footprint | ha | 1,000 |  |
| Overhead |  |  |  |  |  |
|  | Staff | Wages | $ per hour | 25.00 |  |
|  |  | FTEs onshore | # | 4 |  |
|  | Office | Rent | $ per year | 20,000 |  |
|  | Docks | Rent | $ per year | 10,000 |  |
|  | Utilities | Annual cost | $ per year | 5,000 |  |
|  | Insurance | Annual cost | $ per year | 100,000 |  |
|  | Miscellaneous | Annual cost | $ per year | 10,000 |  |